IoT Penetration Testing Methodology

59 Minutes Remaining

Instructions Resources Help  100%

# Exercise 1: IOT Firmware Acquisition, Extraction, Analysis and Emulation

Lab Duration: 10 Minutes

- By default Parrot machine appears. In the Password field type toor and press Enter to login.

- We have a sample firmware package binary for a Netgear IOT device that is often used for practicing IOT hacking. The device model number is WNAP 320, which is a wireless access point for a network. An example of the device is shown in the following image.

- We have downloaded the firmware for you. It is located in the home folder and named firmware.bin.

- The first step when we have a file is to run the file command on it. Launch a Terminal and type sudo file firmware.bin and press Enter. Type toor when prompted for the password and press Enter.

- There is nothing useful for us here, so we continue trying. Type sudo hexdump -c firmware.bin | more and press Enter.

- Now we have qshs that is identified in the hex trace. It is a known signature of the Squashfs file system, so this is a start.

- The next step in the process is to try and extract the file system from this firmware image. Our preferred tool is binwalk, so we will try this. In the terminal window, type man binwalk and press Enter. Take a few minutes to read about the tool. Once you are done, type q to exit the man page.

- In the terminal type sudo binwalk -e firmware.bin and press Enter. However, the main thing is the message that the image has been extracted. We also see that the filesystem is confirmed as Squashfs. Once the command is completed, you will have a directory that represents the extracted filesystem in the Home folder.

- In the terminal type cd _firmware.bin.extracted/squashfs-root and press Enter. Once you are in the directory type ls and press Enter. You will see that the extracted file system is in a squashfs-root directory. We now have a complete and extracted file system, which as you can see, is a Linux variant.

- From here, you can explore it to see the contents. A good place to start is the home directory since it usually contains information about the installed applications. Type cd home and press Enter. Type ls and press Enter to review the results. Since we have a www folder, this can be of interest. Type cd www and Enter, and then type ls and press Enter to view the file contents.

- We have a lot of code here to examine. In most cases, this code is not written securely. The file we want to look at is the boardDataWW.php. Open this in the editor of your choice.

- As you review the code, it is apparent that there is nothing in the way of input validation and boundary checking. This is a clear indication that this application has injection vulnerabilities. Minimize or close the text editor window. There are many files here for you to explore, but we will leave that to you. Next, we will work on the challenges of mounting and booting the firmware with an emulator.

- The one configuration we want to make is point the firmadyne directory to the correct address. Type cd firmadyne and press Enter and open the config file firmadyne.config in the text editor of your choice and set FIRMWARE_DIR to point to the location of the tool, and uncomment the line. Save and close the text editor.

- We will work with the compressed firmware version straight from the download. This is the same file as firmware.bin. Ensure that you are in the firmadyne directory.

- Use the extractor to recover only the filesystem. Ensure that the following are set: no kernel (-nk), no parallel operation (-np), populating the image table in the SQL server at 127.0.0.1 (-sql) with the Netgear brand (-b), and storing the tarball in images. Type sudo python3 sources/extractor/extractor.py -b Netgear -sql 127.0.0.1 -np -nk "WNAP320 Firmware Version 2.0.3.zip" images and press Enter.

- If it works correctly, you will have a tarball in the image directory. Next, enter sudo bash scripts/getArch.sh images/1.tar.gz. Enter the password of firmadyne which is toor when prompted.

- This command is used to identify the architecture of firmware 1 and store the result in the image table of the database.

- Now we want to load the contents of the filesystem for firmware 1 into the database and populate the object and object_to_image tables. Enter sudo python3 scripts/tar2db.py -i 1 -f images/1.tar.gz.

- Next, we need to create the QEMU disk image for firmware 1. Enter sudo bash scripts/makeImage.sh 1. Enter the password of firmadyne which is toor when prompted.

- Now you are ready to setup networking. It should then be available on the network like any other node. Enter sudo bash scripts/inferNetwork.sh 1. Enter the password of firmadyne i.e., toor when prompted.

- Now we are ready to emulate firmware 1 with the inferred network configuration. This will modify the configuration of the host system by creating a Test Access Point (TAP) device and adding a route. Enter sudo bash scratch/1/run.sh.

- This will take some time and execute a chain of instructions as shown in the screenshot. Minimize the terminal window.

- The system should be available over the network. Open a browser, and enter http://192.168.0.100. Netgear WNAP320 access point login page should appear as shown in the screenshot. This is the web interface from the running firmware that is being emulated.

- We can next look at any of the PHP code that we saw when we explored the file system. In the browser, enter http://192.168.0.100/boardDataWW.php. This will bring up another form as shown in the following screenshot.

- Since we had the advantage of reviewing the code, we know that we do not have any input validation, which means that there is a possibility of command injection. We will leave that for you to explore. Hint: You can intercept the query using Burpsuite or any other proxy, and then attempt to enter commands such as cat /etc/passwd.

- Within the Firmadyne tool, we can leverage additional scripts. Open a new terminal and change the directory to firmadyne. Now, enter sudo bash analyses/snmpwalk.sh 192.168.0.100. As the command states, it will enumerate data using SNMP. If you are asked to enter the password, type toor and press Enter.

- We can next enter sudo python3 analyses/webAccess.py 1 192.168.0.100 log.txt.

- There is also a Metasploit interface that will look for vulnerabilities and corresponding exploits. Enter sudo mkdir exploits.

- Change the permissions on the file with sudo chmod +x analyses/runExploits.py.

- Enter sudo python analyses/runExploits.py -t 192.168.0.100 -o exploits/exploit -e x. An example of the output of the command is shown in the following screenshot:

- Once the script finishes, open the file (script.rc) in any text editor. The file can be found in /home/pentester/firmadyne directory.

- As you can see in the above screenshot, the tool is only trying a list of exploits. Therefore, it is not a powerful and effective script. Therefore, it should be used as a reference. Similar to other findings, your ability to research will determine success. The exercise objectives have been achieved.